# UCCNC 4th Axis Generator - README

Target machine: my DIY CC Router

Draft documentation for v2.23

# 1. Purpose

This program is a machine-specific G-code generator for hobby rotary machining. It was developed around a fixed-spindle CNC router with a rotary A axis mounted parallel to machine Y. The design priority is predictable, inspectable toolpaths with explicit operator choices and conservative collision avoidance rather than generic simultaneous multi-axis CAM.

The current workflow supports four operations: ROUND, WAVE, TAPER and GROOVE. Each operation can generate a standalone UCCNC program so cutter changes and probing can be performed deliberately between stages.

# 2. What the program currently does

| Operation | Purpose | Current strategy |
| --- | --- | --- |
| ROUND | Convert square or round stock to a finished cylinder | Indexed no-trench roughing stages followed by a constant-radius spiral cleanup. |
| WAVE | Machine a cosine-style radial profile along Y | Complete 360-degree roughing rings at Y stations, followed by a compensated ball-nose finishing spiral. |
| TAPER | Machine a linear taper between two diameters | Thin-end-first indexed roughing rings with cutter-footprint protection, then one continuous A+Y+Z finishing spiral. |
| GROOVE | Cut one or more helical grooves into an existing profile | Operator explicitly selects ROUND, WAVE or TAPER as the parent surface; multi-pass groove depth and handedness are supported. |

# 3. Important machine assumptions

The values below are defaults for the current build and must not be treated as universal machine settings.

The most recent physical calibration moved A centre X to 778.70 mm, established a usable rotary Y minimum of 1.0 mm, and replaced awkward direct 40120 probing with a spoilboard-based 40 mm Probe 1 workflow.

| Parameter | Current value | Meaning |
| --- | --- | --- |
| A centre X | 778.70 mm | Calibrated machine-X centreline of the rotary axis. |
| Chuck face machine Y | -70.0 mm | Chuck face lies 70 mm before machine Y0. |
| Maximum machine Y | 600.0 mm | Hard upper Y limit used by the generator. |
| Maximum square stock | 80.0 mm | Maximum configured square stock size. |
| Maximum physical stock length | 700.0 mm | Largest physical blank supported by the current fixture. |
| Chuck waste | 20.0 mm | Nominal protected material at the chuck end. |
| Tail waste | 20.0 mm | Nominal protected material at the tail end. |
| A soft-limit magnitude | 3,600,000 deg | Matches the verified UCCNC A-axis soft-limit configuration. |
| Minimum finished diameter | 20.0 mm | Global minimum allowed finished/root diameter. |
| Maximum A speed | 40 RPM | Global software cap used for rotary cutting calculations. |
| Minimum machine Y | 1.0 mm | Hard lower Y limit used for generated rotary motion; Y0 itself triggers UCCNC soft-limit rejection. |
| Probe 1 block height | 40.0 mm | Single-wire touch block placed on the spoilboard for cutter-specific Z probing. |
| Spoilboard to 40120 top | 4.862 mm | Measured vertical step from spoilboard top to the 40120 rotary-bed top. |
| 40120 top to A centre | 65.0 mm | Known rotary centre height above the 40120 bed. |
| Raw Probe 1 to A centre | +29.862 mm | Net correction applied to the raw G53 Probe 1 contact reading. |

# 4. Probe 1 and A-centre reference

Probe 1 is used with the actual fitted cutter. From v2.23 onward the 40.000 mm single-wire Probe 1 block is placed on the spoilboard, where it remains easy to reach even when rotary stock obscures the 40120 bed. The operator enters the raw negative G53 machine-Z value shown immediately after probing. Do not manually subtract the block thickness.

A centre machine Z = raw Probe 1 Z - 40.000 + 4.862 + 65.000 = raw Probe 1 Z + 29.862 mm

The probe conversion uses three fixed dimensions: the 40.000 mm Probe 1 block thickness, the measured 4.862 mm rise from spoilboard to 40120 top, and the 65.000 mm A-centre height above the 40120. Because the actual cutter performs the touch, cutter stick-out is automatically included. The old direct 40120-top probing workflow is no longer the normal operating method.

# 5. Coordinate and phase conventions

XYZ output is generated in G53 machine coordinates.

The rotary axis is called A throughout the program even though it is physically parallel to Y.

UCCNC A-axis rollover on 360 degrees must be OFF.

The original physical A0 orientation is preserved as the phase reference for downstream operations.

For square stock, ROUND assumes the stock starts at A0 in the required original diagonal/corner-up orientation.

WAVE is designed to end at the original A0 physical phase so later operations can use a known angular reference.

# 6. Typical workflow

## 6.1 ROUND

Enter job name, raw stock shape/size, finished diameter, physical stock length and the requested machine-Y cut maximum.

Place the 40 mm Probe 1 block on the spoilboard, probe it with the fitted ROUND cutter, and enter the raw negative G53 machine-Z reading exactly as shown. Do not subtract 40 mm yourself.

For square stock, confirm the original corner-up A0 orientation.

Confirm A-axis rollover is OFF.

Generate and run the ROUND program.

## 6.2 WAVE

ROUND geometry is inherited.

Fit a ball-nose cutter. WAVE cutter diameter/flute count initially follow ROUND but can be edited independently.

Re-probe the 40 mm Probe 1 block on the spoilboard with the actual WAVE cutter and enter the raw G53 Z reading.

Choose wave count, minimum diameter, roughing DOC, finish allowance and cutting parameters.

Generate WAVE only after the geometry and cutter-curvature checks pass.

## 6.3 TAPER

TAPER is locked out when WAVE count is non-zero.

Enter start diameter, end diameter and taper Y start/end.

Probe the 40 mm Probe 1 block on the spoilboard with the actual TAPER cutter and enter the raw G53 Z reading.

Both taper diameters must remain at or above the configured minimum finished diameter.

TAPER roughs from the thin end toward the thick end using indexed rings, then performs a single finishing spiral.

## 6.4 GROOVE

Explicitly select the physical parent profile: ROUND, WAVE or TAPER. There is no auto-detection.

The selected source must pass its own validation before GROOVE is enabled.

Fit the groove cutter, probe the 40 mm Probe 1 block on the spoilboard, and enter the raw G53 Z reading.

Choose cutter diameter, groove depth, max DOC/pass, pitch, number of starts, handedness and start angle.

The groove cutter centre is pulled inward from each parent-profile Y boundary to keep its footprint away from untouched stock.

# 7. Safety and validation philosophy

The program deliberately refuses to generate G-code when machine geometry or operator inputs are inconsistent. Important checks include:

Physical stock length versus chuck-face position, tail waste and requested Cut Y Max.

Hard machine-Y range: Y1.0 to Y600.0. Y0 is physically reachable but is not emitted because UCCNC rejects motion exactly on the configured soft-limit boundary.

Minimum allowed finished/root diameter.

Maximum A RPM and A-axis accumulated travel limit.

ROUND no-trench cutter geometry and residual cleanup limit.

WAVE cutter size versus minimum profile curvature.

TAPER/WAVE mutual exclusion.

GROOVE source validity and root diameter along the selected parent surface.

GROOVE cutter-footprint setback from parent Y boundaries.

These checks do not replace an operator dry run, visual inspection or machine-specific setup verification.

# 8. GROOVE source selection

GROOVE never decides for the operator which surface exists on the workpiece. The base profile is selected explicitly as ROUND, WAVE or TAPER. This is intentional: stale GUI values must not silently choose a machining model.

# 9. Output

Default output directory is currently C:\UCCNC.

Each stage generates a standalone .nc program.

Filenames include job name, operation and timestamp.

Generated headers include key machine and job geometry to make later inspection easier.

# 10. Current development status

v2.23 is the current development build. ROUND and WAVE retain their established machining strategies, while TAPER and GROOVE-on-TAPER remain newer development areas. Machine calibration now uses A_CENTER_X = 778.70 mm, Y_MACHINE_MIN = 1.0 mm and the spoilboard-based Probe 1 conversion described above. The first real cuts after a code or calibration change should be dry-run or air-cut above the work where practical.

# 11. Known limitations / backlog

No saved job recipe/provenance file yet. Returning to a physical part later requires the operator to re-enter geometry that matches the actual workpiece.

A practical lower floor for cutter diameter has not yet been enforced.

Curved/spherical profile support ("ball on a stick") is on the backlog.

TAPER is intentionally a practical fixed-spindle woodworking taper, not a general five-axis surface engine.

Publishing cleanup, portability and more generic machine configuration are still to be done before treating the project as a general-purpose release.

# 12. Before adapting this to another machine

Do not copy the machine constants blindly. Re-measure the rotary centreline, chuck-face relationship, usable Y range, Probe 1 block thickness, spoilboard-to-rotary-bed step, A-centre height above the rotary bed, stock envelope, A-axis speed and soft limits. The code is intentionally explicit about machine geometry because incorrect assumptions can create collisions or leave significant radial stock behind.